Step 1 — Load Energy Table Settings

- Read Energy table.

Step 2 — Apply Energy Table Settings

- Turn off BPM feedback

- Apply Energy table settings

- Mirror out

Step 3 — DCM Piezo Alignment

- Puts DCM Piezo at 5

- Pitch Scan -> intensity peak

- Roll Scan -> BPM x =0

- Pitch Scan -> intensity peak

Step 4 — Mirror Alignment

- Slit scan -> beam center

- Mirror in

- Pitch piezo scan -> BPM y =0

- VDM:Y scan -> peak

- Coupled VFM:Y+VDM:Y -> peak

Step 5 — Enable Feedback Loops

- Turn on H feedback

- DCM piezo pitch scan -> max intensity

- Mirror piezo pitch scan → BPM y = 0

- Turn on V feedback